NED University of Engineering & Technology
Department of Computer and Information Systems Engineering
CS-221 Computer Organization and Design Lab

Session: 14 - Complex Engineering Activity (CEA)		Date: _______________

Student Name: ___________________________   Roll No.: _______________

Seat No.: _______________   Section: _______________

# Lab 14 - Complex Engineering Activity (CEA)

## Simulation of Single-Cycle RISC-V Datapath using Logisim

# 1. Objective

To simulate the complete single-cycle RISC-V datapath by integrating the modules developed in Lab Sessions 8-12: Register File, Instruction Fetch unit, ALU, Data Memory, Main Control Unit, and ALU Control Unit. The objective is to verify correct execution of R-type, I-type, S-type, and B-type RISC-V instructions.

# 2. Problem Statement

The single-cycle RISC-V datapath shown in Figure 14.1 is implemented in Logisim by integrating all previously designed datapath components. The given RISC-V program is loaded into instruction memory, executed step-by-step, and verified through register and data-memory contents.

Figure 14.1 - Single-Cycle RISC-V Datapath

[Attach screenshot of complete simulated datapath here]

# 3. Design & Integration Approach

Register File: Contains thirty-two 32-bit registers with two read ports and one write port.

Instruction Fetch Unit: Uses a 32-bit Program Counter and instruction memory to fetch instructions sequentially.

ALU: Performs add, sub, AND, and OR operations using ALU control signals.

Data Memory: Supports load and store operations controlled by MemRead and MemWrite.

Main Control Unit: Generates ALUSrc, MemToReg, RegWrite, MemRead, MemWrite, Branch, and ALUOp signals.

ALU Control Unit: Decodes ALUOp, funct3, and funct7 fields to generate the ALU operation.

Immediate Generator: Extracts and sign-extends immediates for I-type, S-type, and B-type instructions.

Branch Logic: Uses the Branch signal and ALU Zero flag to decide whether the branch is taken.

# 4. Task 1 - Reference Address & Data Memory Initialisation

Reference address xxx loaded in t0: 0x00000653

Table 14.1 - Partial Main Memory View

| Main Memory Address | Actual Address Used | Contents |
| --- | --- | --- |
| xxx | 0x00000653 | 0x0000FF99 |
| xxx + 4 | 0x00000657 | 0x0000DD88 |
| xxx + 8 | 0x0000065B | 0x0000CC77 |

Screenshot - relevant data memory portion:

[Attach screenshot of data memory initialisation here]

# 5. Task 2 - Instruction Translation & Instruction Memory

Table 14.2 - Code Snippet with Machine Code

| Code Snippet | Machine Code Binary - 32 bits | Machine Code Hex |
| --- | --- | --- |
| lw s1, 0(t0) | 00000000000000101010010010000011 | 0x0002A483 |
| lw s2, 4(t0) | 00000000010000101010100100000011 | 0x0042A903 |
| beq s1, s2, OR | 00000001001001001000111001100011 | 0x01248E63 |
| add t1, s1, s2 | 00000001001001001000001100110011 | 0x01248333 |
| sw t1, 8(t0) | 00000000011000101010010000100011 | 0x0062A423 |
| sub t2, s1, s2 | 01000001001001001000001110110011 | 0x412483B3 |
| sw t2, 12(t0) | 00000000011100101010011000100011 | 0x0072A623 |
| and t3, s1, s2 | 00000001001001001111111000110011 | 0x0124FE33 |
| sw t3, 16(t0) | 00000001110000101010100000100011 | 0x01C2A823 |
| OR: or t4, s1, s2 | 00000001001001001110111010110011 | 0x0124EEB3 |
| sw t4, 20(t0) | 00000001110100101010101000100011 | 0x01D2AA23 |

Screenshot - instruction memory contents after loading program:

[Attach screenshot of instruction memory here]

# 6. Task 3 - Expected Register Contents After Execution

Table 14.3 - Register Contents

| Register | Initial Contents | Expected Final Contents |
| --- | --- | --- |
| t0 | 0x00000653 | 0x00000653 |
| s1 | 0x00000000 | 0x0000FF99 |
| s2 | 0x00000000 | 0x0000DD88 |
| t1 | 0x00000000 | 0x0001DD21 |
| t2 | 0x00000000 | 0x00002211 |
| t3 | 0x00000000 | 0x0000DD88 |
| t4 | 0x00000000 | 0x0000FF99 |

Analysis: s1 = 0x0000FF99 and s2 = 0x0000DD88, so s1 != s2. Therefore, the beq instruction is not taken and the program executes sequentially.

t1 = s1 + s2 = 0x0001DD21

t2 = s1 - s2 = 0x00002211

t3 = s1 AND s2 = 0x0000DD88

t4 = s1 OR s2 = 0x0000FF99

# 7. Task 4 - Single-Step Simulation & Results

The program was executed instruction-by-instruction using Logisim clock stepping. Probes were used to observe PC, ALU output, memory data, register values, and control signals.

Step 1 - lw s1, 0(t0): ALU computes address t0 + 0. Data memory returns 0x0000FF99, which is written to s1.

Step 2 - lw s2, 4(t0): ALU computes address t0 + 4. Data memory returns 0x0000DD88, which is written to s2.

Step 3 - beq s1, s2, OR: ALU subtracts s1 - s2. Since the result is not zero, Zero = 0 and the branch is not taken.

Step 4 - add t1, s1, s2: ALU adds s1 and s2. The result 0x0001DD21 is written to t1.

Step 5 - sw t1, 8(t0): ALU computes t0 + 8 and stores t1 at this data-memory address.

Step 6 - sub t2, s1, s2: ALU subtracts s2 from s1. The result 0x00002211 is written to t2.

Step 7 - sw t2, 12(t0): ALU computes t0 + 12 and stores t2 at this data-memory address.

Step 8 - and t3, s1, s2: ALU performs bitwise AND. The result 0x0000DD88 is written to t3.

Step 9 - sw t3, 16(t0): ALU computes t0 + 16 and stores t3 at this data-memory address.

Step 10 - or t4, s1, s2: ALU performs bitwise OR. The result 0x0000FF99 is written to t4.

Step 11 - sw t4, 20(t0): ALU computes t0 + 20 and stores t4 at this data-memory address.

Final Data Memory Contents

| Address | Final Contents |
| --- | --- |
| t0 + 0 | 0x0000FF99 |
| t0 + 4 | 0x0000DD88 |
| t0 + 8 | 0x0001DD21 |
| t0 + 12 | 0x00002211 |
| t0 + 16 | 0x0000DD88 |
| t0 + 20 | 0x0000FF99 |

Screenshot - register file contents after full program execution:

[Attach screenshot of final register contents here]

Screenshots - various stages of program execution:

[Attach screenshots at various stages of execution here]

# 8. Observations & Discussion

The datapath correctly executed load, store, branch, and R-type instructions. The beq instruction compared s1 and s2; since both values were different, the branch was not taken. The ALU correctly performed addition, subtraction, AND, and OR operations. Store instructions successfully wrote computed values to data memory. The Main Control Unit generated correct control signals for every instruction type, and the ALU Control Unit selected the correct ALU operation.

The Immediate Generator correctly produced immediate values for lw, sw, and beq instructions. Overall, the simulation verified that the integrated single-cycle datapath works correctly.

# 9. Conclusion

The single-cycle RISC-V datapath was successfully simulated in Logisim by integrating the Register File, Instruction Fetch Unit, ALU, Data Memory, Immediate Generator, Main Control Unit, and ALU Control Unit. The program executed correctly, and the final register and memory values matched the expected results. This confirms the proper working of the complete single-cycle RISC-V processor datapath.

# CEP Rubrics

For instructor use

| Criterion | Score |
| --- | --- |
| CPA-1: Depth of Analysis | /4 |
| CPA-2: Fundamental Knowledge | /4 |
| CPA-3: Consequence of Design Change | /4 |
| Uniqueness of Design | /4 |
| Quality of Implementation | /4 |
| TOTAL | /20 |

Remarks: ___________________________________________________________________

Instructor's Signature & Date: ___________________________